# TENNIS BALL SERVING MACHINE

GENERAL PROJECT DESCRIPTION

Design team is asked to design a tennis ball serving machine with rotating discs that will automatically alternate between preset serving positions and be remotely controlled. The number of rotating discs is not limited and may be more then 2. The device must be capable of serving a regulation tennis ball precisely from behind the service line to the opposite service court.  The ball should pass over the net with as little clearance height as possible.  Serves should alternately land in the quarter circle area C and in the quarter circle D.  Team may have up to three tries to hit each of these two circles, but higher points will be achieved with minimum tries. The machine should be a reasonable size (shipping volume less than 500 x 500 x 1000 mm) and easy to assemble and use.  The available fund for material to build the machine is limited to 200KM.  The machine must be completely portable and independent of any external source of energy. Electrical power must be from batteries.  At least 6 balls have to be placed in the machine before these are launched. The operator must actuate machine remotely for each ball independently.

ADDITIONAL DEVICE CONSTRAINTS

- The device must be safe to operate. Combustible or explosive sources of energy could not be used.  The design should reflect consideration for the safety of individuals operating it, holding it, or standing nearby.

- Compression springs should not be used as the main source of energy to place a serve.

- The machine, plus any stand or structure that you may need, must have a shipping volume of less than 50 x 50 x 100cm.  The combined weight of all machine elements including a stand must be less than 25 kg.

- The machine, plus any stand or structure you may need, must have a set-up time of less than 2 minutes.

- If necessary, stabilization weights and sandbags will be provided.

- When serving, the machine and any stand or structure must be placed behind the service line.

- During testing, each machine will have up to three ‘serves’ to hit each landing area C and D, and to get as close as possible to the corner.  For each serve, the distances dC and dD from each corner will be measured, with the ‘best’ values used in the Figure of Merit (FOM).

- The balls should travel as low as possible over the net, like a good tennis serve.  “Zones” E, F, and G will be established by stretching cords above the net at 2 ft intervals.  (There will be some sag in the cords.) Zone H will be above the third cord.  The Figure of Merit equation will be multiplied by a factor of 1.1 for a ball passing through zone E, 1.0 for zone F, 0.9 for zone G, and 0.8 for zone H (anywhere above the top cord). If on any serve a team’s ball strikes a cord, that serve may be re-served with no penalty.

- The machine and any structure or stand must exhibit good workmanship in its construction, be painted or decorated, and have some type of team logo. Team may choose to build an enclosure for the machine.

- The total cost of materials used to construct the machine and any stand or structure is limited to £50. The cost of materials must be completely and accurately documented.

- The machine must exhibit significant design creativity and/or innovation.  (Note: This requirement is intended to prohibit solely commercial or off-the-shelf solutions to the problem.)  The design instructor will make the final determination as to whether this requirement is met.

- When constructing the machine, cutting and drilling operations are permissible, while milling and lathe operations are not.

# FIGURE OF MERIT

The performance of the serving machine will be computed by the following figure of merit (FOM):

Where:

Z 	Multiplier based on above net zone ball passes through
Z=1.1 for zone E, 1.0 for zone F, 0.9 for zone G, and 0.8 for zone H.

C 	Cost of materials in £

dC, dD  Minimum distances (m) from the corner of service court to the ball landing location for best serves.

T	Total number of tries to hit both C and D landing zones. (Six tries maximum.)

W	Total weight of machine plus any stand or structure needed (kg)

A	A number ranging from 0 -4 assessing the workmanship and aesthetic appeal of the machine. Aesthetics also includes the ease and reliability of operation.

E	A number ranging from 0 - 4 assessing the ease of set-up and use of the machine.

DI 	A number ranging from 1 - 4 measuring the creativity and/or innovation of the design.

OI	A number ranging from 0 - 4 assessing the quality of the operating instructions.

# DELIVERABLES

- In addition to designing, constructing, and demonstrating the device, teams must also document and report on their progress as well as provide a final report on the project.

- Teams will provide weekly memo reports documenting their progress.

- Teams will have three design reviews, Term 1 presentation and the final presentations on dates specified in the Timetable.

- A user’s manual for the device detailing assembly, disassembly, and operation of the device must be made available at the time of testing.

- A final report documenting the entire project from initiation to completion will be required at the completion of the project.

- Each team will be required to deliver a computer-based presentation of their project around the time of testing. This will be a sales-oriented presentation designed to convince the instructor that their device is worthy of purchase.